openapi: 3.0.3

info:

title: AgriShield IoT Local API

description: >

Local REST API exposed by the AgriShield IoT module (ESP32).

This API is accessible via WiFi SoftAP and provides read-only access

to sensor data, device status, and controlled configuration endpoints.

version: "1.0.0"

servers:

- url: http://192.168.4.1

description: Local ESP32 SoftAP server

paths:

/health:

get:

summary: Health check

description: Verify that the AgriShield module is alive.

responses:

"200":

description: Module is running

content:

application/json:

schema:

type: object

properties:

status:

type: string

example: OK

uptime_ms:

type: integer

example: 452345

firmware_version:

type: string

example: "1.0.0"

/ping:

get:

summary: Connectivity test

responses:

"200":

description: Pong response

content:

application/json:

schema:

type: object

properties:

pong:

type: boolean

example: true

/status:

get:

summary: Real-time device status

description: Returns the latest sensor values and current alert level.

responses:

"200":

description: Current device status

content:

application/json:

schema:

$ref: "#/components/schemas/DeviceStatus"

/data/log:

get:

summary: Download sensor history

description: >

Returns the complete sensor history recorded by the module.

This endpoint is read-only and represents the source of truth.

responses:

"200":

description: Sensor history JSON

content:

application/json:

schema:

$ref: "#/components/schemas/DataLog"

/config:

get:

summary: Read current configuration

responses:

"200":

description: Current configuration

content:

application/json:

schema:

$ref: "#/components/schemas/DeviceConfig"

post:

summary: Configure module

description: >

Apply or update the operational configuration of the module.

Intended for installer or technician use.

requestBody:

required: true

content:

application/json:

schema:

$ref: "#/components/schemas/DeviceConfig"

responses:

"200":

description: Configuration applied

content:

application/json:

schema:

type: object

properties:

status:

type: string

example: CONFIG_APPLIED

reboot_required:

type: boolean

example: true

components:

schemas:

DeviceStatus:

type: object

properties:

device_id:

type: string

example: AS-001-237

alert_level:

type: string

enum: [GREEN, ORANGE, RED]

example: ORANGE

temperature_air:

type: number

format: float

example: 24.5

humidity_air:

type: number

format: float

example: 68.0

soil_moisture:

type: integer

example: 41

battery_voltage:

type: number

format: float

example: 3.92

solar_charging:

type: boolean

example: true

timestamp:

type: string

format: date-time

example: "2026-02-01T15:12:00Z"

DataLog:

type: object

properties:

device_info:

type: object

properties:

device_id:

type: string

firmware_version:

type: string

crop_profile_id:

type: string

power_status:

type: object

properties:

battery_voltage:

type: number

format: float

battery_level_percent:

type: integer

solar_charging:

type: boolean

last_alert:

type: object

properties:

level:

type: string

enum: [GREEN, ORANGE, RED]

reason:

type: string

timestamp:

type: string

format: date-time

records:

type: array

items:

$ref: "#/components/schemas/SensorRecord"

SensorRecord:

type: object

properties:

timestamp:

type: string

format: date-time

temperature_air:

type: number

format: float

humidity_air:

type: number

format: float

soil_moisture:

type: integer

soil_temperature:

type: number

format: float

DeviceConfig:

type: object

properties:

crop_profile_id:

type: string

example: TOMATO_OPEN_FIELD

sampling_interval_minutes:

type: integer

example: 60

alert_thresholds:

type: object

properties:

humidity_warning:

type: integer

example: 70

humidity_critical:

type: integer

example: 85

temperature_max:

type: integer

example: 35